Spore Lock - The Final Step:  Mould Remediation Instructions

Spore Lock is to be used after full remediation as per accepted guidelines has been conducted.  It is a last step to improve the removal of airborne mold spores by mechanical means.

Mixing:

Add entire contents of Spore Lock bottle to 1 litre of room temperature water.

Concentrate is an eye hazard.  Wear goggles when mixing product

Empty mixture into cold fogger

## Application:

- Use a cold fogger for best application results. Fog entire room at a rate of 1 litre per 800 square feet.

- Set fogger application rate to fine spray

- Apply to all surfaces within room with aim of locking all possible reservoirs of mould

- Wipe off any excess caused by over spraying with microfiber cloth

- Wipe off glass and shiny (chrome) surfaces to prevent water marks

- If porous surfaces such as carpet are in the immediate area, apply liberally to carpet fibers

Personal Protective Equipment

- As per that required for mould remediation.

- Eye protection such as full face mask, N95 cartridges, gloves and Tyvek suit

## Safety and Handling

| A. PRODUCT INFORMATION | CONCENTRATE |  |  | DILUTED WORKING SOLUTION |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Composition: | 3 (trimethoxy)silylpropyldi-methyloctadecyl ammonium chloride diluted to 42% active ingredients by weight with methanol |  |  | 3.0% by weight of the concentrate in water |  |  |
| Appearance: | Clear to yellow |  |  | Clear to yellow tint |  |  |
| Hazard Classification | Flammable liquid due to methanol |  |  | Not flammable |  |  |
| B. STORAGE |  | CONCENTRATE |  |  | DILUTED WORKING SOLUTION |  |
| Flammability: |  | Flammable |  |  | Not Flammable |  |
| Shelf Life: |  | 12 months in closed container. |  |  | Up to 6 months in closed containers with no water evaporation or external contaminants. |  |
| Recommended Storage Temp: |  | Ambient.3,4 |  |  | Ambient3 Protect from Freezing |  |
| D. HANDLING |  | CONCENTRATE | EMERGENCY FIRST AID |  |  | DILUTED WORKING SOLUTION |
| Skin Contact: |  | Can cause irritation. Wear gloves and appropriate skin protection | Flush exposed area with water |  |  | No effect on skin.1 |
| Eye Contact: |  | Severe effect on eyes, can cause blindness. Wear goggles or a face shield to prevent contact with eyes. | Flush eyes with water for 15 minutes. Obtain medical help. |  |  | Slight irritation. Poses no hazard from incidental contact.1 |
| Inhalation: |  | Prolonged or repeated exposure to undiluted material may have an intoxicating effect. Provide adequate ventilation and use an appropriate respirator | Remove to fresh air. |  |  | No effect. |
| Ingestion: |  | Large doses can cause blindness or be fatal. | If swallowed, drink a large quantity of water and obtain medical attention. |  |  | Poses no hazard from ingestion incidental to commercial handling. |

*Always wear standard safety apparel and protective devices when handling chemicals*

Full SDS is available for review at www.moldscience.net